Formal international recognition of the International Terrestrial Reference System (ITRS)

Claude Boucher

CGEDD

V04 10 dec 2008

Background

The International Terrestrial Reference System (ITRS) is presently the recommended Terrestrial Reference System (TRS) for the whole geoscience community, through a resolution adopted by the International Union of Geodesy and Geophysics (IUGG) during its General Assembly of Perugia in 2007:

«The International Union of Geodesy and Geophysics

Considering the increasing importance of geodetic reference systems in Geosciences, and more generally in numerous scientific or technical activities, such as satellite  navigation systems or geo-information,

Noting  the IUGG Resolution 2 and IAG Resolution 1, both adopted in 1991 at the Vienna General Assembly, defining the Conventional  Terrestrial Reference System (CTRS)

Recognizing the quality of the work done by several  IAG services (IERS, IGS, ILRS, IVS, IDS,…)  to  actually  realize these systems and provide regular access for  numerous users within and  beyond the geoscience community,

Endorses the definition of a Geocentric Terrestrial Reference System (GTRS) as a “System of geocentric space-time coordinates within the framework of General Relativity, co-rotating with the Earth and related to Geocentric Celestial Reference System by a spatial rotation which takes into account the Earth orientation Parameters”, in agreement with the IAU resolution B1.3 2000,

Endorses the definition of the International Terrestrial Reference System (ITRS) as the  specific GTRS for which the orientation is operationally maintained in continuity with past international agreements (so-called BIH orientation)

Furthermore adopts the ITRS as preferred system for any scientific application and urges other communities such as geo-information, or navigation to do the same.

 Note:

This resolution upgrades the content of the IUGG Resolution 2 of 1991, by adopting the designation of GTRS in preference to the term of Conventional Terrestrial Reference System and by explicitly endorsing the ITRS as the unique preferred GTRS.”

The ITRS is also recognized by the International Astronomical Union (IAU).

The fundamental realizations and access of ITRS are provided by several scientific services under the umbrella of the International Association of Geodesy (IAG), one of the member associations of IUGG.

 The International Earth Rotation and Reference Systems Service (IERS) is, since its establishment in 1988, in charge of the primary realization of ITRS, by an optimal combined use of space geodetic techniques. Several successive solutions were computed and released to the international community, labelled International Terrestrial Reference Frames: ITRFyy (ITRF88, up to the current ITRF2005).

 Furthermore, each primary space geodetic technique is  now organized in the frame of an international scientific service, also under the IAG : IVS for VLBI, ILRS for laser ranging, IDS for DORIS and IGS for GNSS (GPS, GLONASS and soon Galileo). Each of these services not only contributes to the IERS activities by providing input data, but also uses the IERS products to compute and release their own operational products. In particular, they use the ITRF solutions to express their products in the ITRS. This is true in particular for satellite orbits (expressed in the rotating terrestrial system). Such products are widely used by the geodetic and geophysical community, mostly for GPS, as provided by IGS.

These primary realizations are also densified and disseminated through regional, national and local terrestrial geodetic networks (permanent markers or active stations, such as GPS stations providing differential corrections). A wide coordination is ensured regionally by ad hoc committees, also under the IAG umbrella: EUREF for Europe, SNARF and CORS for North America, SIRGAS for South America…

Another community also provides  realizations and accesses to ITRS, with a rapidly growing importance, namely the Satellite Navigation community (GPS, GLONASS, EGNOS, Galileo, …) commonly designated by GNSS. An upper lever coordination of this community (providers as well as users) is now existing through the International Committee for GNSS (ICG), linked to UN through the UN Office for Outer Space Afairs (UNOOSA), which provides the secretariat for ICG (see http://www.unoosa.org/oosa/SAP/gnss/icg.html).

Concerning the GNSS community, it is important to notice that USA and EU have formally recognized in their agreement about GPS and Galileo interoperability that the individual reference frame of each navigation system should be as close as possible realize the ITRS. 

Although for instance, aerial navigation with ICAO, cartography with ICA, meteorological station coordinates with WMO, or hydrography with IHO have formally recognized and adopted WGS84. More precisely, it means that each of these organizations has adopted resolutions recommending the use of WGS84.

At this point, one must realize that the issue is only a wording issue: the WGS84 system designates a full set of geodetic standards, in which successive realizations of a unique TRS has been provided. The very initial one was based on Transit Doppler measurements, and used to be referred to the international TRS currently accepted in 1984, as defined by the Bureau International de l’Heure (BIH). This system is exactly the one which received the name of ITRS in 1988. Later the main WGS84 realizations were based on GPS, and the most recent solutions are in agreement with ITRF at a few centimetre level. To summarize, WGS84 products (as concerning TRS issues) must be considered as various realizations of ITRS. To name it WGS84, in place of ITRS, is confusing, and even wrong. It is satisfactory to see that the USA/EU document about GNSS used the word ITRS. Organizations such as ICAO, IHO or ICA should do the same, by modifying their past resolutions.

On the other hand, in Europe, the European Commission  and EuroGeographics are adopting ETRS89, following the EUREF recommendations. Here, the issue is different: ETRS89 is a TRS which is not identical to ITRS but is defined exactly from ITRS by a mathematical formula. A clear understanding of these points is important, regarding in particular the adoption of the INSPIRE  directive.

Findings

From the previously described background situation, one can derive the following statements:

a) ITRS is well defined, adopted and realized, but only within the geoscience community, through a resolution of the relevant international scientific organization, the IUGG. 

b) Various system designations are adopted in recommendations, standards or rules of several communities of practice: ITRS, WGS84, ETRS89…which implies potential confusions or non inter-operability

c) The existence, quality, perenity or reliability of ITRS realizations are presently ensured by various scientific groups or even individuals, without unified and inter-governmental guarantee, like the situation of time scales a few years ago, before the involvement of BIPM.

Proposals

Therefore, we formulate the following proposals:

1. To establish a new ISO standard which will endorses the ITRS as the unique preferred terrestrial reference system for all domains of activity, and which defines general rules for the realization of ITRS through frames compliant with the ISO standard.

2. To establish an international and inter-governmental governing structure which ensures the existence, quality, consistency and  perenniality of the various realizations of ITRS.

3. To ensure the proper clarifications in terminology and explanations of ITRS with regard to other terms such as WGS84, ETRS89, SNARF/CORS, …

These three proposals are presented with more details thereafter.

Proposal 1: ISO standard

The objective is to prepare an ISO standard and make it approved according to the ISO procedures. The two main points are:

a) the exact content of the standard, and specifically its level of detail;

b) the actual establishment of the standardization procedure. A baseline idea is that a few member bodies of ISO submit such a proposal, with the IAG support (through GGOS). At this point, France (AFNOR) is ready to do so.

Proposal 2: International governance

This point is important and must be investigated with care. Several ways can be considered. We suggest three tracks:

a) to create a body related to an existing UN organization;

b) to create a body related to an inter-governmental agreement, such as the General Conference on Weights and Measures (CGPM), with potential involvement of BIPM;

c) to create a new body through an ad hoc international or intergovernmental agreement, possibly in the frame of GEO or under the auspice of a relevant UN agency.

Proposal 3: Link with other system designations

As previously reported, some TRSs had been or are being adopted by some communities as recommended ones. The adoption of ITRS should be ultimately consistent with the practices of these communities, in one way or another. The system designations considered in this issue are:

· WGS84. In this case, the proposal is to view WGS84 as the cover designation for a specific realization of ITRS. This should be accepted by the satellite navigation community (the ICG mentioned before should play the leading role), civil aviation (ICAO), hydrography (IHO), meteorology (WMO), cartography and geo-information (ICA), and ultimately the military community (USA or NATO).

· ETRS89. As this system is defined precisely with regards to ITRS by EUREF, this should raise no problem if the issue is clearly documented.

· Other regional systems and frames. The necessity to have these systems and frames related to ITRS in a well-defined and clearly expressed way is emphasized here.

Contact: claude-boucher@club-internet.fr
